# 1. Skipton Site

### HR Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC01 | 192.70.20.194 | 255.255.255.240 | 192.70.20.193 | Check Packet Tracer |
| Laptop02 | 192.70.20.195 | 255.255.255.240 | 192.70.20.193 | Check Packet Tracer |
| PC03 | 192.70.20.196 | 255.255.255.240 | 192.70.20.193 | Check Packet Tracer |
| Laptop04 | 192.70.20.197 | 255.255.255.240 | 192.70.20.193 | Check Packet Tracer |

### Sales Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC05 | 192.70.20.2 | 255.255.255.224 | 192.70.20.1 | Check Packet Tracer |
| PC06 | 192.70.20.3 | 255.255.255.224 | 192.70.20.1 | Check Packet Tracer |
| PC07 | 192.70.20.4 | 255.255.255.224 | 192.70.20.1 | Check Packet Tracer |

### IT Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC08 | 192.70.20.100 | 255.255.255.224 | 192.70.20.97 | Check Packet Tracer |
| DNS Server | 192.70.20.98 | 255.255.255.224 | 192.70.20.97 | Check Packet Tracer |
| HTTP Server | 192.70.20.99 | 255.255.255.224 | 192.70.20.97 | Check Packet Tracer |

# 2. Wrocław Site

### HR Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC68 | 192.70.20.210 | 255.255.255.240 | 192.70.20.209 | Check Packet Tracer |
| PC69 | 192.70.20.211 | 255.255.255.240 | 192.70.20.209 | Check Packet Tracer |

### Sales Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC70 | 192.70.20.34 | 255.255.255.224 | 192.70.20.33 | Check Packet Tracer |
| PC71 | 192.70.20.35 | 255.255.255.224 | 192.70.20.33 | Check Packet Tracer |
| PC72 | 192.70.20.36 | 255.255.255.224 | 192.70.20.33 | Check Packet Tracer |
| PC73 | 192.70.20.37 | 255.255.255.224 | 192.70.20.33 | Check Packet Tracer |

### IT Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| Laptop77 | 192.70.20.130 | 255.255.255.224 | 192.70.20.129 | Check Packet Tracer |
| Laptop78 | 192.70.20.131 | 255.255.255.224 | 192.70.20.129 | Check Packet Tracer |
| Laptop79 | 192.70.20.132 | 255.255.255.224 | 192.70.20.129 | Check Packet Tracer |
| Smartphone | 192.70.20.133 | 255.255.255.224 | 192.70.20.129 | Check Packet Tracer |

# 3. Greenwich Site

### HR Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC98 | 192.70.20.226 | 255.255.255.240 | 192.70.20.225 | Check Packet Tracer |
| Laptop99 | 192.70.20.227 | 255.255.255.240 | 192.70.20.225 | Check Packet Tracer |

### Sales Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC100 | 192.70.20.66 | 255.255.255.224 | 192.70.20.65 | Check Packet Tracer |
| PC101 | 192.70.20.67 | 255.255.255.224 | 192.70.20.65 | Check Packet Tracer |
| PC102 | 192.70.20.68 | 255.255.255.224 | 192.70.20.65 | Check Packet Tracer |

### IT Office

| Device | IP Address | Subnet Mask | Gateway | MAC Address |
| --- | --- | --- | --- | --- |
| PC104 | 192.70.20.162 | 255.255.255.224 | 192.70.20.161 | Check Packet Tracer |
| PC105 | 192.70.20.163 | 255.255.255.224 | 192.70.20.161 | Check Packet Tracer |
| PC106 | 192.70.20.164 | 255.255.255.224 | 192.70.20.161 | Check Packet Tracer |

# 4. VLAN configuration table

| Device | VLANs to Configure |
| --- | --- |
| Switch0 | VLAN 10, VLAN 20, VLAN 30 |
| Switch05 | VLAN 10, VLAN 20, VLAN 30 |
| Switch06 | VLAN 10, VLAN 20, VLAN 30 |

Use these VLAN names:

| VLAN ID | VLAN Name | Department |
| --- | --- | --- |
| 10 | Sales | Sales Office |
| 20 | IT | IT Office |
| 30 | HR | HR Office |

# 5. Router serial interface addressing

Following the router order in your assignment, use Router0 for Skipton, Router1 for Wrocław, and Router2 for Greenwich if those names match your topology.

| Device / Interface | IP Address | Subnet Mask |
| --- | --- | --- |
| Router0 – Se0/1/0 | 192.70.20.241 | 255.255.255.252 |
| Router1 – Se0/1/0 | 192.70.20.242 | 255.255.255.252 |
| Router0 – Se0/1/1 | 192.70.20.245 | 255.255.255.252 |
| Router2 – Se0/1/1 | 192.70.20.246 | 255.255.255.252 |
| Router1 – Se0/1/1 | 192.70.20.249 | 255.255.255.252 |
| Router2 – Se0/1/0 | 192.70.20.250 | 255.255.255.252 |

The serial interfaces on these routers are Serial0/1/0 and Serial0/1/1 (not Se0/0/0 and Se0/0/1). Each address is on the interface cabled to the neighbour shown in show ip ospf neighbor. On Router2 (Greenwich) the ports are the opposite way round to Router0 and Router1: Se0/1/1 faces Skipton and Se0/1/0 faces Wrocław.

# 6. Router LAN interface addressing

| Router | Subinterface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Router0 – Skipton | G0/0.10 | 192.70.20.1 | 255.255.255.224 |
| Router0 – Skipton | G0/0.20 | 192.70.20.97 | 255.255.255.224 |
| Router0 – Skipton | G0/0.30 | 192.70.20.193 | 255.255.255.240 |
| Router1 – Wrocław | G0/0.10 | 192.70.20.33 | 255.255.255.224 |
| Router1 – Wrocław | G0/0.20 | 192.70.20.129 | 255.255.255.224 |
| Router1 – Wrocław | G0/0.30 | 192.70.20.209 | 255.255.255.240 |
| Router2 – Greenwich | G0/0.10 | 192.70.20.65 | 255.255.255.224 |
| Router2 – Greenwich | G0/0.20 | 192.70.20.161 | 255.255.255.224 |
| Router2 – Greenwich | G0/0.30 | 192.70.20.225 | 255.255.255.240 |

# 7. Routing protocol table

| Router | Protocol | Network Advertised | Configuration Command |
| --- | --- | --- | --- |
| Router0 | OSPF | 192.70.20.0/24 | network 192.70.20.0 0.0.0.255 area 0 |
| Router1 | OSPF | 192.70.20.0/24 | network 192.70.20.0 0.0.0.255 area 0 |
| Router2 | OSPF | 192.70.20.0/24 | network 192.70.20.0 0.0.0.255 area 0 |

Open the browser from PC-1, type in the URL: www.coventrylab.ac.uk in the address bar and open the web page. Take a snapshot of what comes up on the webpage or landing page and paste it in the text box below:

Show workings of how you arrived at the IP Addresses of your Network Design

### IP Addressing and VLSM Calculations

The network was allocated the IP address block 192.70.20.0/24 for the three sites: Skipton, Wrocław and Greenwich. I used Variable Length Subnet Masking (VLSM) to divide the network into smaller subnets for the Sales, IT and HR departments, as well as the router-to-router connections.

1. Sales and IT Subnets (/27)

Sales and IT were assigned a /27 subnet mask of 255.255.255.224.

- Total IP addresses: 2⁵ = 32

- Usable host addresses: 32 − 2 = 30

- Block size: 256 − 224 = 32

Therefore, each /27 subnet contains 30 usable host addresses. The network address increases in blocks of 32.

| Site/Department | Network Address | Usable Host Range | Broadcast Address | Default Gateway |
| --- | --- | --- | --- | --- |
| Skipton Sales | 192.70.20.0/27 | .1 – .30 | .31 | 192.70.20.1 |
| Wrocław Sales | 192.70.20.32/27 | .33 – .62 | .63 | 192.70.20.33 |
| Greenwich Sales | 192.70.20.64/27 | .65 – .94 | .95 | 192.70.20.65 |
| Skipton IT | 192.70.20.96/27 | .97 – .126 | .127 | 192.70.20.97 |
| Wrocław IT | 192.70.20.128/27 | .129 – .158 | .159 | 192.70.20.129 |
| Greenwich IT | 192.70.20.160/27 | .161 – .190 | .191 | 192.70.20.161 |

2. HR Subnets (/28)

HR required fewer host addresses, so I used a /28 subnet mask of 255.255.255.240.

- Total IP addresses: 2⁴ = 16

- Usable host addresses: 16 − 2 = 14

- Block size: 256 − 240 = 16

The network address therefore increases in blocks of 16.

| Site/Department | Network Address | Usable Host Range | Broadcast Address | Default Gateway |
| --- | --- | --- | --- | --- |
| Skipton HR | 192.70.20.192/28 | .193 – .206 | .207 | 192.70.20.193 |
| Wrocław HR | 192.70.20.208/28 | .209 – .222 | .223 | 192.70.20.209 |
| Greenwich HR | 192.70.20.224/28 | .225 – .238 | .239 | 192.70.20.225 |

3. Router-to-Router Connections (/30)

I used /30 subnets for the serial connections between the routers because each point-to-point link requires only two usable IP addresses.

- Subnet mask: 255.255.255.252

- Total IP addresses: 2² = 4

- Usable host addresses: 4 − 2 = 2

- Block size: 256 − 252 = 4

| Connection | Network Address | Router 1 IP | Router 2 IP | Broadcast Address |
| --- | --- | --- | --- | --- |
| Skipton–Wrocław | 192.70.20.240/30 | 192.70.20.241 | 192.70.20.242 | 192.70.20.243 |
| Skipton–Greenwich | 192.70.20.244/30 | 192.70.20.245 | 192.70.20.246 | 192.70.20.247 |
| Wrocław–Greenwich | 192.70.20.248/30 | 192.70.20.249 | 192.70.20.250 | 192.70.20.251 |

4. DNS and HTTP Server Addresses

The DNS and HTTP servers were placed in the Skipton IT subnet, 192.70.20.96/27. I reserved the first usable address for the default gateway and assigned the next two addresses to the servers.

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Skipton IT Gateway | 192.70.20.97 | 255.255.255.224 | — |
| DNS Server | 192.70.20.98 | 255.255.255.224 | 192.70.20.97 |
| HTTP Server | 192.70.20.99 | 255.255.255.224 | 192.70.20.97 |

The DNS server was configured with an A record that maps www.coventrylab.ac.uk to 192.70.20.99, the HTTP server's address.

Conclusion

By using VLSM, I allocated address ranges according to the needs of each department and router connection. Each department has its own subnet and default gateway, allowing the network to be organised clearly and reducing address wastage. The plan uses 252 of the 256 addresses (about 98%). Only 192.70.20.252/30 remains unallocated, so there is almost no room for growth without a larger address block.